# Hackney Carriage and Private Hire Vehicle Testing Manual

## Examination and testing requirements in addition to those forming part of the test to MOT standards

Minor Failures do not require suspension of the vehicle. Arrangements for the vehicle to be re-assessed should be made with the relevant office identified below.

Major Failures will result in the suspension of the vehicle until the defect is rectified. Arrangements for the vehicle to be re-assessed should be made with the relevant office identified below.

| 1 | Wheels/Tyres |  |  |  |
| --- | --- | --- | --- | --- |
|  | Method of Inspection Check tread on all tyres Check condition of spare wheel and tyre (where accessible). | Reason for Failure Tread does not meet 1.6mm depth across the full width of the tyre Does not meet legal requirement. Exemption: where there is no provision for a spare wheel | Failure Type Major – suspension of vehicle on failure Major – suspension of vehicle on failure | Retest Location Moorhey Street Moorhey Street |
| 2 | Seatbelts |  |  |  |
|  | Method of Inspection Inspect all non-obligatory seatbelts (where fitted) to the same MOT test standard as obligatory seatbelts | Reason for Failure A non-obligatory seatbelt that doesn’t meet MOT test standard for an obligatory seatbelt | Failure Type Major – suspension of vehicle on failure | Retest Location Moorhey Street |
| 3 | Seating Seating security and condition |  |  |  |
|  | Method of Inspection Check all seats are properly cushioned and free from damage. Check minimum seat width per passenger | Reason for Failure Seat cushion badly worn, damaged, or insecure. Base or back rest cushion worn to the extent that the frame protrudes. Seating dirty or stained. Unauthorised seat covers. Seat covers are not permitted unless they are compatible with airbags. Approval must be sought by Licensing prior to the installation of any seat covers. The minimum accepted width for passenger seats is 400mm. The measurement point will be taken from the front of the seat cushion. For bench seats, the total width required to provide the minimum space per passenger is 1200mm. | Failure Type Minor Minor Minor Major – suspension of vehicle on failure Photos to be taken of any of the above issue(s) Major – must licence for less passengers or withdraw app Vehicles licensed before 21 October 2025 that don’t meet this requirement have grandfather rights | Retest Location Moorhey Street Moorhey Street Moorhey Street Moorhey Street |
| 4 | Doors Locks, hinges, panelling, etc. |  |  |  |
|  | Method of Inspection Check door seals, handles, bonnet catches and support struts for security and condition In respect of Hackney Carriages – automatic door lock operation and function | Reason for Failure Door seals missing/damaged/incorrectly fitted or not creating a proper seal Check passenger doors lock when footbrake is applied and that warning lamps come on | Failure Type Minor Minor | Retest Location Moorhey Street Moorhey Street |
| 5 | Glass & Windows Condition of windows, glass or other transparent material |  |  |  |
|  | Method of Inspection Check the condition of full windscreen for cracks/surface damage. Check that all windows that are designed to open can do so. Check for security, wind and water tightness. From inside the vehicle, check that all windows and screens are free from defect or other objects, which may excessively impair vision and that they conform to the manufacturer’s specification. | Reason for Failure A crack, significant surface damage in glass or other transparent material. Tinted windows – can be manufacturer fitted or film as long as they meet the following: Front windscreen minimum 75% light transmission Front side glass minimum 70% light transmission Remaining glass or rear side windows (excluding rear window) minimum 20% light transmission All windows will not open/close or are difficult to open/close. All windows will not open completely to manufacturers specification. Interior of vehicle is not wind and watertight Unauthorised stickers or objects attached to windows or screens. Damaged/peeling/poorly fitted tinted windows. | Failure Type Major – suspension of vehicle on failure Minor Major – suspension of vehicle on failure Major – suspension of vehicle on failure Minor Minor – to be removed or replaced (if replaced transmission to be rechecked) | Retest Location Moorhey Street Moorhey Street Moorhey Street Moorhey Street Licensing Office Moorhey Street |
| 6 | Passenger Communication |  |  |  |
|  | Method of Inspection From inside the vehicle check that any passenger can communicate with the driver without restriction. Check that intercom system (where fitted) works correctly | Reason for Failure Panelling between front and rear compartment giving no means of adequate access for audible communication Panel not transparent Intercom system not working/faulty/unclear in operation | Failure Type Major – suspension of vehicle on failure Major – suspension of vehicle on failure Major – suspension of vehicle on failure | Retest Location Licensing Office Licensing Office Moorhey Street |
| 7 | Interior Lights |  |  |  |
|  | Method of Inspection Check operation of interior lights with door shut. Open each door individually and check the operation of the interior lights. Operate internal switches to check correct operation of interior lights | Reason for Failure Interior light does not operate when any one door is opened (where fitted) Interior light not giving off sufficient light so as to cause difficulty in vision to either rear or front seated passengers Passenger light switches not clearly marked (hackney carriage) Interior light does not operate when switched on | Failure Type Minor Minor Minor Minor | Retest Location Moorhey Street Moorhey Street Moorhey Street Moorhey Street |
| 8 | Heater |  |  |  |
|  | Method of Inspection Examine heater for operation, security and condition Check condition of heater hoses | Reason for Failure Noisy operation Leaking matrix or hoses Insecure installation Heater faulty or inoperative Controls or switches faulty | Failure Type Minor Minor Minor Minor Minor | Retest Location Moorhey Street Moorhey Street Moorhey Street Moorhey Street Moorhey Street |
| 9 | Load security Luggage boot area, boot lid/tailgate |  |  |  |
|  | Method of Inspection Check for presence/security/condition and weather tightness of boot lid/tailgate support struts/mechanism and weather seal Check that they support the boot lid/tailgate safely in the open position and seal correctly in the closed position. Check condition and presence of weather seals. | Reason for Failure Support struts/mechanism missing/insecure/leaking or damaged Boot lid/tailgate is not self-supporting in the fully opened position Damage or missing boot door/tailgate weather seal | Failure Type Major – suspension of vehicle on failure Major – suspension of vehicle on failure Major – suspension of vehicle on failure | Retest Location Moorhey Street Moorhey Street Moorhey Street |
| 10 | Condition of body and paintwork (see detailed bodywork/paintwork requirements) |  |  |  |
|  | Method of Inspection Examine interior and exterior of body to identify corrosion, cracks or damage and condition of paintwork | Reason for Failure Cracked panels/sections Paintwork not a uniform colour match, blistered or eroded Poor quality repair | Failure Type Minor/major depending on scope of issue – for all in this section Photos to be taken of issue | Retest Location Moorhey Street |
| 11 | Interior Panels Security & condition of panels, security & condition of sun visors |  |  |  |
|  | Method of Inspection Examine all interior panels for security and condition. | Reason for Failure Headlining insecure, excessively dirty, stained or poorly repaired Any raised item or hole on floor which could be a potential trip hazard. Any holes where seats have been removed must be covered. | Failure Type Minor/Major – depending on scale of dirt or damage Photos to be taken of issue Major – suspension of vehicle | Retest Location Moorhey Street Moorhey Street |
| 12 | Cleanliness of vehicle |  |  |  |
|  | Method of Inspection Examine vehicle for interior and exterior cleanliness, including seating, carpets or matting and luggage area. Examine the underside of the vehicle and engine compartment for cleanliness | Reason for Failure Worn carpets/matting, which could be a potential hazard to passengers Dirty seating/interior/trim/carpets/matting or loose objects accumulated on seat/floor/scuttle panel in luggage boot External body of vehicle excessively dirty Foul odour in the vehicle Engine or underside of vehicle in an excessively dirty condition | Failure Type Major – suspension of vehicle Minor/Major – depending on scale of dirt or damage. Major – suspension of vehicle Major – suspension of vehicle Major – suspension of vehicle Photos to be taken of any visual issues | Retest Location Moorhey Street Moorhey Street Moorhey Street Moorhey Street Moorhey Street |
| 13 | Licence Plate |  |  |  |
|  | Method of Inspection Examine rear of the vehicle to check the plate is displayed in the authorised plate holder and fixed in the plate holder using the clips provided to allow it to be easily removed by an authorised officer. The plate should be securely fixed to the vehicle in such a way that neither it, nor the number plate are obscured, and that both are 100% visible. Cable ties are not an acceptable means of fixing a plate to a vehicle or the plate holder | Reason for Failure Plate not fixed to the vehicle Plate fixed to the vehicle but not in the proper manner i.e., not in the plate holder with clips, or any use of cable ties, Velcro, double-sided tape etc. Licence plate not clearly legible due to dirt, debris, oil etc. Licence plate damaged | Failure Type (for all items) Advisory – to replace/correct via Licensing Office | Check Location (for all items) Licensing Office before collection of plate |
| 14 | Stickers, signage & advertising |  |  |  |
|  | Method of Inspection Private Hire: Examine vehicle and ensure the Oldham Council issued mandatory door sign is displayed on the top half of the rear passenger doors. Ensure the vehicle is displaying the Oldham Council issued front windscreen identification/licence disc Ensure the vehicle is displaying the relevant operator disc in the front windscreen Hackney Carriage: Examine vehicle and ensure the Oldham Council issued mandatory door sign is displayed on the top half of the front driver & passenger doors. Ensure the vehicle is displaying the Oldham Council issued front windscreen identification/licence disc ALL Any advertising displayed on vehicle must have been approved by Oldham Council, including the use of any Operator door signs Smoking stickers | Reason for Failure Door signs not displayed Door signs displayed in wrong place Door signs poorly fitted i.e., not straight or correctly adhered to the vehicle and showing peeling, significant bubbling/rippling Windscreen licence disc not displayed Operator signage missing Door signs not displayed Door signs displayed in wrong place Door signs poorly fitted i.e., not straight or correctly adhered to the vehicle and showing peeling, significant bubbling/rippling Windscreen licence disc not displayed Operator door signs in use that are no longer permitted Advertising on vehicle that has not been approved No smoking sticker(s) (minimum of 1) must be displayed in the vehicle | Failure Type (for all items) Advisory – to replace/correct via Licensing Office | Check Location (for all items) Licensing Office before collection of plate |